REQUEST FOR PROPOSAL
SPECIFICATION OF SPECIFIED GOODS

Procurement No:	RFP-18-G002-21

## Specification

### Background

### The main objective of the project is to build outer island copra sheds for targeted islands to replace the old buildings that are currently not safe for storing mounting amount of copra on those islands. The following islands are targeted for the project:

### Aranuka, Abemama, Nikunau, Tab-North, Nonouti and Onotoa.

### Phase 1 will start off with the procurement of materials and building of copra sheds for Abemama, Tab North, Nikunau and Onotoa.

### Phase 2 of the project will be the construction of the remaining copra sheds which will be carried out on Aranuka and Nonouti once additional funds secured.

### Work is planned to be commenced in early 2021 but delayed due to correction of costing/amount to be committed in the project.

### Requirements

All supporting documentation must be in English.  The following is a full list of required documents required for submission by tenderers:

- Business Registration certificate

- Business Licence

- Company/Business bank statement (most recent, no later than 1 month old)

- Tax clearance

- Detailed Work plan

- Company references from clients

- Warranty of at least 6 months

- Certificate of Compliance form (signed)

### Installation services

Services required includes, procuring of building materials, carpentry and instalments of the pre-fabricated building for copra sheds

### Delivery Time

End of July 2021 to end of December 2021. The completion timeline of the project is subject to the Tenderer’s work plan but this is the baseline.

## Description of the Goods

All items listed below are the Building Materials for 1 set Prefab Copra Shed. The required number of copra shed to be established is four (4)

| PREFAB COPRA SHED (1 Unit) |  |  |  |
| --- | --- | --- | --- |
| Dimension: 20 M*10 M*5.7 M |  |  |  |
| Total Area (M2) | 200.00 |  |  |
| Design Standard(Design load): roof dead load:0.3KN/m2, roof live load:0.3KN/m2, wind load:0.6KN/m2 |  |  |  |
| Item | Unit | Qty. | Specification |
| Main Steel Frame System |  |  |  |
| Main structure | ton | 4.74 | H section frameH300*180*6*8，Q345 Steel ,galvanized |
| Roof purlin | ton | 1.01 | C160*60*20*2.0,galvanized |
| Wall purlin | ton | 1.61 | C160*60*20*2.0,galvanized |
| Intensive bolt | set | 120.00 | Grade 10.9 |
| Anchor bolt | pcs | 42.00 | M24, steel Q235, processing |
| Ordinary bolt | set | 600.00 | Galvanized bolt |
| Bracing System |  |  |  |
| Cross brace | ton | 0.21 | Φ20 round steel bar, steel Q235, galvanized |
| Knee brace | ton | 0.10 | L50*4 angle steel Q235, galvanized |
| Column bracing | ton | 0.21 | Φ20 round steel bar, steel Q235, galvanized |
| Tie bar | ton | 0.40 | Φ89*3.0, steel Q235,galvanized |
| Batter brace | ton | 0.38 | Φ32*2，Φ12round steel bar, steel Q235, galvanized |
| Wall & Roof System |  |  |  |
| Roof Panel | M2 | 222.48 | 0.5mm color steel plate, Model V840 |
| Roof cladding board | M2 | 358.00 | 0.5mm color steel plate, Model V900 |
| Trim plate | M | 180.00 | 0.6mm color plate, angle Alu. |
| fittings & accessories | M2 | 580.48 | Nails、glue etc. |
| Doors&windows |  |  |  |
| Door | set | 1.00 | electric rolling door (4000mm*4000mm) |
| Window | m2 | 19.20 | PVC window |

## Tenderer’s References

### Relevant similar deliveries carried out in the last five years

Please, provide information on each delivery for which your firm/entity, either individually as a corporate entity or as one of the major companies within an association, was legally contracted.

| Goods delivered | Buyer | Contact details | Value |
| --- | --- | --- | --- |