REQUEST FOR PROPOSAL
SPECIFICATION OF WORKS

Procurement No: 16 - W001 - 26

Request for Proposal (RFP) to Supply and Construct a Half-round Barn Shed

for Kiritimati Island Materials Recovery Facility

## Specification

### Background

Kiribati has a Deposit & Refund recycling system, called the Kaoki Maange, operating in the capital, Tarawa, and it is now intended to extend the system to Kiritimati Island. Land has been allocated in Kiritimati Island for the setting up of a Materials Recovery Facility (MRF) so that the Kaoki Maange can be operated in the Line Islands. A shed is required in which to place the can baling machine so that cans can be baled for export, and batteries packed for export and recycling. Previously in Tarawa, the Kaoki Maange operated for twenty years from a simple half-round barn made in New Zealand for farmers. These barns are widely used in New Zealand, and are easy to construct in remote locations with minimal equipment, being designed as hay barns that may be built  in very remote mountain country to hold winter stock feed. These type of sheds require minimal foundation blocks just under the steel framework, and a concrete slab can be poured after construction, where required. The old Tarawa barn had a protective coating of Coloursteel treatment, and this has proven very resistant to corrosion in an exposed location at the Port of Betio, even after 20 years. One downside of the Tarawa shed was that it was open-ended, and so rain could blow in onto electrical equipment and machinery, causing corrosion over time, and thus malfunction. This tender requires a half-round barn shed with doors at each end, so that in rainy weather and at night the building can be closed up.

Construction is very simple and the building is designed so that  minimal lifting and crane work is required due to local constraints. Also, as noted, a full foundation slab for the entire building is not required for the construction to take place, and a floor slab can be poured after the building is erected. Construction is the matter of a few days. The site for the new MRF is at the Port of Kiritimati and so there is little cartage required in Kiritimati Island to get from the Port to the construction site, a matter of a few hundred metres at most.

### Requirements

The Contract will be to supply and construct the shed. The shed will have to be sourced and imported from overseas to the Port of Kiritimati Island, and transported to the site for construction. The distance from the Port to the expected site is less than 1 km. Everything required to build the shed should be included in the price, being materials, labour and all transportation costs to get the shed and materials onto the construction site.

All supporting documentation must be in English.

The shed must meet the following specifications:

Cladding materials; 'Coloursteel' or 'Colourbond' or similar production finish, steel thickness 0.5mm or greater;

Flashings to be steel thickness 0.5mm or greater with the same coating as the cladding where possible;

Steel lattice truss must be 40 x 10 FMS steel  or greater,  and  Galvanised with a uniform coating of 85 Microns minimum thickness;

Any timber elements must be to NZ standard timber treatment process level 3.2 or greater;

All screws and bolts used must be galvanised

Sliding Doors on rollers on each end with the same cladding specification as the main structure so the building can be closed up;

Gutters on each side full length of the building;

All flashings to suit the building;

Six 3.6m bays, total length 21.6m

Building overall dimensions:  21.6m long, with the building 8.5m wide and 4.85m high:

Two water tanks of 5,000 litres each, one for each side, connected to the guttering with all required pipe-work supplied plus valves and fittings at the bottom of the tanks.

### Related services

The successful bidder will also be required to construct the shed on a site which has been designated by LMD. The successful bidder must supply all concrete materials, including cement, steel work, gravel etc to make the foundations for the steelwork, and make the foundation blocks for the steelwork. Locally sourced sand and aggregate can be used. Bidders must include in their price the cost to make a concrete pad of 150mm thickness under the footprint of the building, and must include the cost of all materials and labour to do so, including steel mesh to comply with MISE building permit requirements.

The contractor will provide all technical services required to gain a  building permit from MISE on Kiritimati Island, and cooperate with MISE to complete the building permit process. The cost of the permit will be paid by KSWMP.

### Project Time & Final Delivery

Shipping delivery to Kiritimati Island is four times per year on international carrier PDL. The delivery needs to be completed by November  2026, and construction completed by December 31st 2026

## Description of the Works

Here, list all items to be Tendered

(This part may be replaced by a Procuring Entity or proprietary Contractor description)

| Pos. | Description | Delivery Time (to be Tendered) | Price (to be Tendered) |
| --- | --- | --- | --- |
| 1 | Half-round barn, 21.6m x 8.5m x 4.85m high, as a kit set building, complete with gutter and end doors, to specified quality; | By October 2026 |  |
| 2 | Delivery from country of manufacture to Kiritimati Island | By November 2026 |  |
| 3 | Two 5,000 litre water tanks, including tap fittings at the outlets | By November 2026 |  |
| 4 | Construct half-round barn at Tennessee near the Port of Kiritimati and hook up water tanks | By December 31st 2026 |  |

## Tenderer’s References

### Relevant similar deliveries carried out in the last five years

Please, provide information on each delivery for which your firm/entity, either individually as a corporate entity or as one of the major companies within an association, was legally contracted.

| Works delivered | Reference | Contact details | Value |
| --- | --- | --- | --- |